END OF SEMESTER EXAMINATIONS

2ND SEMESTER 2022/2023 ACADEMIC YEAR

DATE: JULY 2023

COURSE CODE: CE 301

COURSE TITLE: ELECTRONIC DEVICES AND CIRCUITS

LECTURER’S NAME: DR. MAHAMUDA ALHAJI MAHAMADU DURATION: 3 HOURS

APPENDICES

|  | COURSE OUTLINE (MAIN TOPICS) | QUESTION NO. |
| --- | --- | --- |
| MajorTopic-1 | PN Junction Diode | 1,6a, 7c, 7d, 9d, 10d |
| MajorTopic-2 | Intrinsic and Extrinsic Semiconductors | 6b, 8c |
| MajorTopic-3 | Electronic Circuit | 5, 7b,9b, 10b, |
| MajorTopic-4 | Semiconductor devices | 6d, 10d |
| MajorTopic-5 | Diodes and Applications of Diodes | 3,6c, 9a, 10a |
| MajorTopic-6 | Bipolar Junction Transistors | 2, 4, 8d,9c, 10c |
| MajorTopic-7 | Transistor Biasing and Stabilization | 8a |
| MajorTopic-8 | Field Effect Transistors | 7a, 8b |

PART A (UNDERSTANDING)

INSTRUCTIONS: Part A contains FIVE questions. Answer ALL questions.

Questions

1.  Based on the electrical conductivity all the materials in nature are classified as insulators, semiconductors, and conductors. In your own words explain why PN junction diodes are made from semiconductor materials.

| Major Topic PN Junction Diode | Blooms Designation UN | Score 5 |
| --- | --- | --- |

2.  Intrinsic semiconductor has very limited applications as they conduct very small amounts of current at room temperature. The current conduction capability of intrinsic semiconductor can be increased significantly by adding a small amounts impurity to the intrinsic semiconductor. By adding impurities it becomes impure or extrinsic semiconductor. This process of adding impurities is called as doping. With the aid of a diagram, explain how the N-type extrinsic semiconductor is formed.

| Major Topic Principles of Semiconductor Devices | Blooms Designation UN | Score 5 |
| --- | --- | --- |

3. Write briefly on how the reverse breakdown voltage of a diode may decrease with increasing temperature?

| Major Topic Diodes and Applications of Diodes | Blooms Designation UN | Score 5 |
| --- | --- | --- |

4. In a tabular format, compare the P channel and the N channel JFET

| Major Topic FET Transistors | Blooms Designation UN | Score 5 |
| --- | --- | --- |

5.  Find the power delivered to an element at t=20ms if the current i=50cost50пt A and the voltage v=2i V

| Major Topic Electronic Circuit | Blooms Designation UN | Score. 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

PART B[APPLICATION AND ANALYSIS]

INSTRUCTIONS: Part B contains THREE questions. Attempt any TWO questions.

Question 6.

a) With the aid of a diagram explain what happens when a PN Junction diode is reversed biased.

| Major Topic PN Junction Diode | Blooms Designation AP | Score 7 |
| --- | --- | --- |

b) In n-type Ge, the donor concentration corresponds to 1 atom per 107  Ge atoms. Assume that the effective mass to the electron equals one-half of the true mass. At room temperature, how far from the edge of the conduction band is the Fermi level?

| Major Topic Intrinsic and Extrinsic Semiconductors | Blooms Designation AN | Score 7 |
| --- | --- | --- |

c) In Fig. 1, the silicon controlled rectifier (SCR) has a trigger voltage of 0.7 V. Calculate the supply voltage that turns on the crowbar. Ignore zener diode resistance.                        
                                                                Fig. 1

| Major Topic Diodes and Applications of Diodes | Blooms Designation AP | Score 6 |
| --- | --- | --- |

d) With the aid of a diagram explain the mechanism by which an electron contributes to conductivity in a semiconductor material such silicon

| Major Topic Semiconductor devices | Blooms Designation AN | Score 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

Question 7

- For the JFET in Fig. 2, find VDS and VGS, given that ID = 5 mA

Fig. 2

| Major Topic Field Effect Transistors | Blooms Designation AP | Score 7 |
| --- | --- | --- |

- For the series voltage regulator shown in Fig.3, calculate (i) output voltage and (ii) Zener current.

Fig. 3

| Major Topic Electronic Circuits | Blooms Designation AN | Score 7 |
| --- | --- | --- |

c) Compute and sketch the room temperature characteristics for a PN silicon junction with the following characteristics. W=10-3 cm, NA=1 x 1019/cm3, ND=2 x 1016/cm3, A=10-4 cm2. Assume that: Dh=8cm2/sec, De=25cm2/sec, ni=1.0 x 1010/cm3

| Major Topic PN junction Diode | Blooms Designation AP | Score 6 |
| --- | --- | --- |

d) In your own word explain why silicon preferred to germanium in the manufacturing of semiconductor devices such as a PN Junction Diode?

| Major Topic PN Junction Diode | Blooms Designation AN | Score 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

Question 8

- A base bias circuit in Fig. 4 is subjected to an increase in temperature from 25°C to 75°C. If β = 100 at 25°C and 150 at 75°C, determine the percentage change in Q-point values ( VCE and IC) over this temperature range. Neglect any change in VBE and the effects of any leakage current.

### Fig 4

| Major Topic Transistor Biasing and Stabilization | Blooms Designation AP | Score 7 |
| --- | --- | --- |

b) In Fig. 5,  an n-channel JFET biased by potential divider method, it is desired to set the operating point at ID = 2.5 mA and VDS = 8V. If VDD = 30 V, R1 = 1 MΩ and R2 = 500 kΩ, find the value of RS. The parameters of JFET are IDSS = 10 mA and VGS (off) = – 5 V.                             
			   								                                               Fig 5

| Major Topic Field Effect Transistors | Blooms Designation AN | Score 7 |
| --- | --- | --- |

### c) With the aid of a diagram describe how the p-type extrinsic semiconductor is formed

| Major Topic Intrinsic and Extrinsic Semiconductors | Blooms Designation AP | Score 6 |
| --- | --- | --- |

d)	Explain the operations of the BJT as a Switch

| Major Topic Bipolar Junction Transistor | Blooms Designation AN | Score 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

PART C [EVALUATING AND CREATING]

INSTRUCTIONS: Part C contains TWO questions. Answer ONE question.

Question 9

- The bridge rectifier shown in Fig. 6 uses silicon diodes. Find (i) d.c. output voltage (ii) d.c. output current. Use simplified model for the diodes.

Fig. 6

| Major Topic Diodes and Applications of Diodes | Blooms Designation CR | Score 7 |
| --- | --- | --- |

- The current through a 0.1H inductor is i(t)=10t e-5t. Find the voltage across the inductor and energy stored in it.

| Major Topic Electronic Circuit | Blooms Designation EV | Score 7 |
| --- | --- | --- |

- Determine the Q point of the transistor circuit shown in Fig. 7.

Fig. 7

| Major Topic BJ Transistors | Blooms Designation CR | Score 6 |
| --- | --- | --- |

d)  In your own words briefly explain the term peak inverse voltage in PN in PN Junction diodes

| Major Topic PN Junction Diode | Blooms Designation EV | Score 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

Question 10

a) For the circuit shown in Fig.8, find the output D.C. voltage

Fig. 8

| Major Topic Diodes and Applications of Diodes | Blooms Designation CR | Score 7 |
| --- | --- | --- |

- Find the power delivered to an element at t=5ms if the current i=5cost60пt A and the voltage

| Major Topic Electronic Circuits | Blooms Designation EV | Score 7 |
| --- | --- | --- |

### c) Determine VCB in the transistor circuit shown in Fig. 10. The transistor is of silicon and has β = 150.

.

Fig. 10

| Major Topic BJT Transistors | Blooms Designation CR | Score 6 |
| --- | --- | --- |

- Explain in your own words the term doping. why doping is important.

| Major Topic Semiconductor devices | Blooms Designation EV | Score 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

END OF QUESTION PAPER