END OF SEMESTER EXAMINATIONS

2ND SEMESTER 2022/2023 ACADEMIC YEAR

DATE: JULY 2023

COURSE CODE: EEE 306

COURSE TITLE: MICROPROCESSOR I&II

LECTURER’S NAME: HARRY WARDEN

DURATION: 3 HOURS

APPENDICES

|  | COURSE OUTLINE (MAIN TOPICS) | QUESTION NO. |
| --- | --- | --- |
| MajorTopic-1 | Application of Microprocessors | 1 |
| MajorTopic-2 | General Concept of Microprocessors | 2 |
| MajorTopic-3 | Computer Software | 3 |
| MajorTopic-4 | Evolution of Intel Microprocessors | 4 |
| MajorTopic-5 | RISC and CISC Architecture | 5 |
| MajorTopic-6 | Microprocessor Architecture Types | 6a, b, c, d |
| MajorTopic-7 | General Concept of Microprocessors | 7a, b, c, d |
| MajorTopic-8 | 8086 Microprocessors | 8a, b, c, d |
| MajorTopic-9 | Assembly language Statement/Instruction | 9a,b.c.d & 10a,b,c,d |

PART A (UNDERSTANDING)

INSTRUCTIONS: Part A contains FIVE questions. Answer ALL questions.

Questions

1.  Outline ten (10) practical applications of microprocesses.

| Major Topic Application of Microprocesses | Blooms Designation UN | Score 5 |
| --- | --- | --- |

2.  In your opinion, why is it important for every engineer to learn how to use or have knowledge about microprocesses.

| Major Topic General Concept of Microprocessors | Blooms Designation UN | Score 5 |
| --- | --- | --- |

3.  Differentiate between Source Code and Object Code in Computer Software.

| Major Topic Computer Software | Blooms Designation UN | Score 5 |
| --- | --- | --- |

4. List ten (10) Intel microprocesses according to their Names, Bit size and Year of release.

| Major Topic Evolution of Intel Microprocessors | Blooms Designation UN | Score 5 |
| --- | --- | --- |

5.  Differentiate between RISC (reduced instruction set computer) and CISC (complex instruction set computer).

| Major Topic RISC And CISC Architecture | Blooms Designation UN | Score. 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

PART B[APPLICATION AND ANALYSIS]

INSTRUCTIONS: Part B contains THREE questions. Attempt any TWO questions.

C

C

B

A

Question 6.

- Identify Figure A and B indicated above.

| Major Topic Microprocessor Architecture Types | Blooms Designation AP/AN | Score 7 |
| --- | --- | --- |

- Differentiate between Figure A and B.

| Major Topic Microprocessor Architecture Types | Blooms Designation AP/AN | Score 7 |
| --- | --- | --- |

- Identify the part labelled C.

| Major Topic Microprocessor Architecture Types | Blooms Designation AP/AN | Score 5 |
| --- | --- | --- |

- Explain the role/function of the part labelled C.

| Major Topic Microprocessor Architecture Types | Blooms Designation AP/AN | Score 6 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

Question 7

A factory in the production of liquid detergents needs a device to automatically control the agitator of one of its remote mixing tanks.

- As electronics engineer of that factory, what device would you recommend.

| Major Topic General Concept of Microprocessor | Blooms Designation AP | Score 7 |
| --- | --- | --- |

- Give reasons for your choice.

| Major Topic General Concept of Microprocessor | Blooms Designation AN | Score 7 |
| --- | --- | --- |

- Name the components that make up the device above.

| Major Topic General Concept of Microprocessor | Blooms Designation AP | Score 5 |
| --- | --- | --- |

- Explain the functions of each component named in question (c) above.

| Major Topic General Concept of Microprocessor | Blooms Designation AN | Score 6 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

Question 8

Figure A

- Identify Figure A above.

| Major Topic 8086 Microprocessor | Blooms Designation AP | Score 7 |
| --- | --- | --- |

- Enumerate the functions of the following pins: A19/S6, A18/S5, A17/S4, A16/S3.

| Major Topic 8086 Microprocessor | Blooms Designation AN | Score 7 |
| --- | --- | --- |

- Which pin is used to indicate the transfer of data over the higher order (D15-D8) data bus.

| Major Topic 8086 Microprocessor | Blooms Designation AP | Score 6 |
| --- | --- | --- |

- What are these pins: AD15-AD0.

| Major Topic 8086 Microprocessor | Blooms Designation AN | Score 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

PART C [EVALUATING AND CREATING]

INSTRUCTIONS: Part C contains TWO questions. Answer ONE question.

Question 9

INC	COUNT;

MOV TOTAL, 28;

ADD   AH, BH;

AND   FASK, 120;

ADD   SCORE, 5;

MOV   AL, 5;

- Write the meaning of the instruction or comment on the program instruction

MOV   TOTAL, 28;

| Major Topic Assembly language Statement | Blooms Designation CR/EV | Score 7 |
| --- | --- | --- |

- Write the meaning of the instruction or comment on the program instruction.

ADD   AH, BH;

| Major Topic Assembly language Statement | Blooms Designation CR/EV | Score 7 |
| --- | --- | --- |

- Write the meaning of the instruction or comment on the program instruction.

AND   FASK, 120;

| Major Topic Assembly language Statement | Blooms Designation CR/EV | Score 6 |
| --- | --- | --- |

- Write the meaning of the instruction or comment on the program instruction.

INC   COUNT;

| Major Topic Assembly language Statement | Blooms Designation CR/EV | Score 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

Question 10

- Write an assembly language program instruction to compare registers AL and BL.

| Major Topic Assembly language Statement | Blooms Designation CR/EV | Score 7 |
| --- | --- | --- |

- Write an assembly language program instruction to move count in AX.

| Major Topic Assembly language Statement | Blooms Designation CR/EV | Score 7 |
| --- | --- | --- |

- Write an assembly language program instruction to clear the counter.

| Major Topic Assembly language Statement | Blooms Designation CR/EV | Score 6 |
| --- | --- | --- |

- CODE SEGMENT

START: MOV   AX, DATA;

MOV   DS, AX;

What does the highlighted instruction mean?

| Major Topic Assembly language Statement | Blooms Designation CR/EV | Score 5 |
| --- | --- | --- |

TOTAL SCORE: 25 MARKS

END OF QUESTION PAPER